Case Study: Shoreline changes in Cedar Key, Florida using Google Earth Pro and DSAS

Abstract

The perpetuation of climate change and sea level rise have led to concerns in shoreline dynamics in the Gulf of Mexico. Shoreline dynamics in areas of coastal development have been intensely studied, however many under-developed shorelines have yet to be analyzed. In this study we used eight NAIP (National Agriculture Imagery Program) aerial images, from 1994 to 2019, of our study area near Cedar Key, FL.  The cloud-free images were collected during relatively similar mean river discharge levels and during (mostly) the same season. We assessed the shoreline changes using the ArcMap extension DSAS (Digital Shoreline Analysis Systems). The DSAS analysis is a transect- based approach and is used to quantify shoreline changes on a linear ocean shoreline. From this analysis we have been able to determine the greatest areas of impact and speculate on possible factors that may be contributing to an escalated shoreline change rate during this brief time frame.

1. Introduction

Shorelines changes can occur due to multiple factors including anthropogenic, natural, hurricane intensity, and sea level rise (Yu et al., 2011). The combination of these processes can influence erosion and accretion. These shoreline changes may affect the resilience to storm surges including flooding and species diversity implications (Desantis et al., 2007). It was observed by USGS (US Geological Survey) that shoreline changes along the Gulf of Mexico, specifically in Florida, were relatively steady between the 1800s and 1990s (Morton et al., 2005). Since then, the Gulf of Mexico coastline, with its low relief geomorphology particularly along the west coast of Florida, has been noted to be vulnerable to coastal erosion (Geselbracht et al., 2011).

1.2 Reason for effort

In the mid-1960s the US Army Corps of Engineers constructed spoil islands as part of the cross Florida barge canal project. These spoil islands consist of a straight line of islands perpendicular to the coast (Fredrick et al., 2019). An example of habitat analysis was conducted by Vitale (2019) and investigated how some of these spoil islands were and and are severely eroded or currently inundated, thus reducing habitat for animals. Derrick Key is an example of a spoil island that was clearly visible in aerial photographs in 1982 and now the island is completely submerged (in 2016 photography). Major shoreline differences are noticeably observed in the 34 years, time between the imagery, for this specific spoil island.

Figure- Island degradation of Derrick Key in the Cedar Keys, Florida from 1982 (left) to 2016 (right), (Fredrick et al., 2019).

2. Materials and methods

2.1 Study Area

Our study area is located on the west-central Florida coastline. The selected shoreline is a small barrier island called Deer Island. Deer island is a privately owned uninhabited island approximately 8 miles north of the main villages of Cedar Key, Florida. Historically, Native Americans intermittently inhabited Deer Island for thousands of years. Early Florida settlers were reported to live and camp on the island as well. The 1800 Florida census registered only 4 people to have identified this island as their home.  There is a cabin near the south of the island depicted on a 1951 USGS Cedar Key Quadrangle map (USGS, 1955). This island is specifically located in the Big Bend Aquatic Seagrass Preserve and connects with the Lower Suwannee National Wildlife Refuge (http://www.beachrealtyfla.com/DeerIsland.htm). Deer Island is approximately 90 acres of total area and consists of 25 upland acres and 20 wetland acres with elevations as high as 14 feet. The island is densely forested with large pines, cedars, palms, oaks, palmettos and many more plant species (https://www.privateislandsonline.com/united-states/florida/deer-island). The shoreline attributes reported on Deer island is about 0/8+/- mile of Gulf of Mexico white sand beach and approximately 0.8 +/- mile of waterfront facing the mainland ( https://images1.loopnet.com/d2/Z4L1-alqEsAlhPT_YJ25N8OMkXU3L_mAPAZYXiq2OVg/document.pdf).

Figure- Location of Deer Island, Florida. A) Map of the entire state of Florida; B) Zoomed into map scale of 2.3758 to location; C) Zoomed into map scale of 0.03 to Deer Island with a scale bar in kilometers

2.3 Imagery selection process

Selecting cloud-free imagery of a specific location can be time consuming. Since our study location is not populated nor contains any historic landmarks, there are not frequent aerial or satellite passes of this area.

| Date | Median River Discharge (cfs) | Observed weather | Metadata |
| --- | --- | --- | --- |
| January 20, 1994 | Station ID= 02323500 Value= 9710 | Avg Temp (F)- 38.15 Precipitation (inches)- 0.00 Max Wind Speed (MPH)- 12 | Found DOQ in Earth Explorer Entity ID:DI00000000018672 Entity ID DI00000000018672 Map Name CEDAR KEY Acquisition Date 1994/01/21 State FL Quadrant NW Status Historical Product Group 3.75-MIN CIR Production System GIS/MAGIC Production Date 1996/08/14 Primary Source Date 1994/01/21 Coordinate System Universal Transverse Mercator (UTM) Coordinate Zone 17 Photo Source(s) NAPP 7000 057 Cell ID 134545 ODB Prod ID 18672 Resolution 1 Version 1 Band Type RGB DOQ Format Band Interleaved by Pixel File (BIP) Standards Version DOQ 12/96 Standard Spec. Primary H Datum North American Datum of 1983 XY Unit Meters Submitting Agency Western Mapping Center (WMC) Oversight Agency Western Mapping Center (WMC) |
| December 30, 1998 | Station ID= 02323500 Value= 6370 | Avg Temp (F)- 48.75 Precipitation (inches)- 0.00 Max Wind Speed (MPH)- 16 | Found as DOQ in Earth Explorer Entity ID:DI00000001164809 Entity ID DI00000001164809 Map Name CEDAR KEY Acquisition Date 1998/12/31 State FL Quadrant NW Status Recommended Version Product Group 3.75-MIN CIR Production System DVx.x xx/xx0Vx.x xx/xx(USGS) Production Date 2001/09/07 Primary Source Date 1998/12/31 Coordinate System Universal Transverse Mercator (UTM) Coordinate Zone 17 Photo Source(s) NAPP 11018 073 Cell ID 134545 ODB Prod ID 1164809 Resolution 1 Version 2 Band Type RGB DOQ Format Band Interleaved by Pixel File (BIP) Standards Version DOQ 12/96 Standard Spec. Primary H Datum North American Datum of 1983 XY Unit Meters Submitting Agency Western Mapping Center (WMC) Oversight Agency Western Mapping Center (WMC) |
| November 02, 2007 | Station ID= 02323500 Value= 2350 | Avg Temp (F)- 66.76 Precipitation (inches)- 0.00 Max Wind Speed (MPH)- 14 | NAIP Entity ID N_2908356_NW_17_1_20071102 State FL Agency USDA Vendor USDA-FSA-APFO Map Projection UTM Projection Zone 17N Datum NAD83 Resolution 1.000000000000000 Units METER Number of Bands 3 Sensor Type CLR Project Name 200707_FLORIDA_NAIP_1X0000M_CLR Acquisition Date 2007/11/02 |
| September 19, 2010 | Station ID= 02323500 Value= 4240 | Avg Temp (F)- 77.57 Precipitation (inches)- 0.00 Max Wind Speed (MPH)- 15 | NAIP Entity ID M_2908356_NW_17_1_20100919 State FL Agency USDA Vendor USDA-FSA-APFO Map Projection UTM Projection Zone 17N Datum NAD83 Resolution 1.000000000000000 Units METER Number of Bands 4 Sensor Type CNIR Project Name 201004_FLORIDA_NAIP_1X0000M_CNIR Acquisition Date 2010/09/19 |
| October 13, 2013 | Station ID= 02323500 Value= 8200 | Avg Temp (F)- 71.83 Precipitation (inches)- 0.00 Max Wind Speed (MPH)- 10 | NAIP Entity ID M_2908356_NW_17_1_20131013 State FL Agency USDA Vendor USDA-FSA-APFO Map Projection UTM Projection Zone 17N Datum NAD83 Resolution 1.000000000000000 Units METER Number of Bands 4 Sensor Type CNIR Project Name 201305_FLORIDA_NAIP_1X0000M_CNIR Acquisition Date 2013/10/13 |
| November 12, 2015 | Station ID= 02323500 Value= 6070 | Avg Temp (F)- 66.68 Precipitation (inches)- 0.00 Max Wind Speed (MPH)- 9 | NAIP Entity ID M_2908356_NW_17_1_20151112 State FL Agency USDA Vendor USDA-FSA-APFO Map Projection UTM Projection Zone 17N Datum NAD83 Resolution 1.000000000000000 Units METER Number of Bands 4 Sensor Type CNIR Project Name 201504_FLORIDA_NAIP_1X0000M_UTM_CNIR Acquisition Date 2015/11/12 |
| October 26, 2017 | Station ID= 02323500 Value= 7990 | Avg Temp (F)- 54.68 Precipitation (inches)- 0.00 Max Wind Speed (MPH)- 9 | NAIP Entity ID M_2908356_NW_17_1_20171026 State FL Agency USDA Vendor USDA-FSA-APFO Map Projection UTM Projection Zone 17N Datum NAD83 Resolution 1.000000000000000 Units METER Number of Bands 4 Sensor Type CNIR Project Name 201710_FLORIDA_NAIP_1X0000M_UTM_CNIR Acquisition Date 2017/10/26 |
| November 10, 2019 | Station ID= 02323500 Value = 5190 | Avg Temp (F)- 57.42 Precipitation (inches)- 0.00 Max Wind Speed (MPH)- 7 | NAIP Entity ID M_2908356_NW_17_060_20191110 State FL Agency USDA Vendor USDA_FSA_APFO Map Projection UTM Projection Zone 17N Datum NAD83 Resolution 0.600000000000000 Units METER Number of Bands 4 Sensor Type CNIR Project Name 201911_FLORIDA_NAIP_0X6000M_UTM_CNIR Acquisition Date 2019/11/10 |

Table- https://www.fsa.usda.gov/Assets/USDA-FSA-Public/usdafiles/APFO/support-documents/pdfs/fourband_infosheet_2017.pdf

wunderground.com/history/daily/us/fl/gainesville/KGNV/date/2012-1-8

| Sensor Type | Bands and wavelength (µm) |
| --- | --- |
| CLR/ RGB | Blue 400–500 Green 500–600 Red 600–700 |
| CNIR | Blue 400–500 Green 500–600 Red 600–700 Near Infrared 800–900 |

Table- National Agriculture Imagery Program (NAIP) aerial imagery

Table

(https://www.fsa.usda.gov/Assets/USDA-FSA-Public/usdafiles/APFO/support-documents/pdfs/fourband_infosheet_2017.pdf)

2.3. Digital Shoreline Analysis System (DSAS)

Figure - DSAS generates transects that are cast perpendicular to the reference baseline at a user-specified spacing alongshore.  There are no restrictions on where the reference baseline is drawn, it may be positioned completely to one side of the shoreline data or be placed between the historical shoreline positions.  DSAS measures the distance between the baseline and each shoreline intersection along a transect, and combines date information, and positional uncertainty for each shoreline, to  generate the following change metrics (https://www.usgs.gov/centers/whcmsc/science/digital-shoreline-analysis-system-dsas?qt-science_center_objects=0#qt-science_center_objects):

Distance measurements:

- Shoreline Change Envelope (SCE)

- Net Shoreline Movement (NSM)

Statistics:

- End Point Rate (EPR)

- Linear Regression Rate (LRR)

- Weighted Linear Regression Rate (WLR)

Supplemental statistics for Linear and Weighted regression:

- Confidence Interval (LCI/WCI)

- Standard Error (LSE/WSE)

- R-squared (LR2/WR2)

3. Results

4. Discussion and conclusion

References

U.S. Geological Survey, 1955, USGS 1:24000-scale Quadrangle for Cedar Key, FL 1955: U.S. Geological Survey. (https://www.sciencebase.gov/catalog/item/5a8a3ffbe4b00f54eb3ec75e)